# Modbus Simulator Setup & Read/Write Testing

# 1.Software and Requirements

Windows 10 computer

Docker Desktop

WSL 2

Git

Python 3.13

PyModbus development repository

PowerShell

# 2. Installation and Initial Setup

Step 1: Install Docker Desktop

Docker Desktop was installed on the Windows computer. Docker provides an environment for running containerized applications.

Step 2: Verify Docker Installation

Docker was tested using the standard Hello World container.

docker run hello-world

The successful 'Hello from Docker!' output confirmed that Docker was working correctly.

Step 3: Enable Hardware Virtualization

Docker Desktop initially reported that virtualization support was not detected. Hardware virtualization was enabled in BIOS/firmware.

VM Monitor Mode Extensions: Yes

Virtualization Enabled in Firmware: Yes

Second Level Address Translation: Yes

Data Execution Prevention Available: Yes

Step 4: Install and Configure WSL 2

Windows Subsystem for Linux was installed because Docker Desktop uses WSL 2 on this setup.

wsl --install --no-distribution

After restarting Windows, WSL was configured with Default Version 2.

Step 5: Install/Verify Git

Git was verified and added temporarily to the current PowerShell PATH.

$env:Path += ";C:\Program Files\Git\cmd" git --version

# 3. Download PyModbus Source Code

Step 6: Clone the PyModbus Repository

The official PyModbus repository was cloned locally using the development branch.

git clone -b dev https://github.com/pymodbus-dev/pymodbus.git

Repository location:

C:\Users\User\pymodbus

Step 7: Verify the Repository

The repository was checked to confirm the dev branch and clean working tree.

cd C:\Users\User\pymodbus git status

# 4. Install PyModbus

Step 8: Install PyModbus in Editable Mode

The local PyModbus source was installed in editable mode. This ensured that the development simulator code and the installed package matched.

py -m pip install -e .

Step 9: Install/Verify aiohttp

aiohttp was installed/verified because the simulator uses its web server component.

py -m pip install aiohttp

py -c "from aiohttp import web; print('aiohttp.web OK'); print(web)"

# 5. Locate the Simulator

Step 10: Locate the Simulator Example

The repository contains the simulator example at:

C:\Users\User\pymodbus\examples\simulator.py

The example script is a demonstration that can start a server, run demo client calls, and stop. For a continuously running simulated PLC, the installed simulator executable was used.

Step 11: Check Simulator Help

The simulator executable was checked to identify its configuration options.

& "$env:LOCALAPPDATA\Packages\PythonSoftwareFoundation.Python.3.13_qbz5n2kfra8p0\LocalCache\local-packages\Python313\Scripts\pymodbus.simulator.exe" --help

# 6. Start the Simulated PLC

Step 12: Run the Simulator

The simulator was started with the device_try configuration and server mode. Keep this PowerShell window open during testing.

& "$env:LOCALAPPDATA\Packages\PythonSoftwareFoundation.Python.3.13_qbz5n2kfra8p0\LocalCache\local-packages\Python313\Scripts\pymodbus.simulator.exe" --modbus_device device_try --modbus_server server

Step 13: Verify the Server Started

The simulator displayed messages confirming the Modbus and HTTP services.

Modbus server started on ('0.0.0.0', 5020) HTTP server started on (0.0.0.0:8081) Server listening.

The Modbus TCP service listens on port 5020.

# 7. Verify Network Connectivity

Step 14: Test TCP Port 5020

Open a second PowerShell window while the simulator is running and test the local Modbus port.

Test-NetConnection 127.0.0.1 -Port 5020

Expected important result:

TcpTestSucceeded : True

This confirms that the local computer can reach the simulated PLC through TCP port 5020.

# 8. Understand the Simulator Configuration

Step 15: Locate setup.json

The simulator's internal configuration file was located here:

C:\Users\User\pymodbus\pymodbus\server\simulator\setup.json

The device_try configuration defines simulated data areas, including coils, discrete inputs, holding registers and input registers.

Holding register size: 63000

Coil size: 63000

Discrete input size: 63000

Input register size: 63000

Address 10 is included as a writable address in the device_try configuration

# 9. Create the Python Read/Write Client

Step 16: Create test_read_write.py

The following Python client connects to the simulated PLC, reads a register, writes 1234 to register 10, and reads it again.

import asyncio from pymodbus.client import AsyncModbusTcpClient async def main(): client = AsyncModbusTcpClient("127.0.0.1", port=5020) await client.connect() print("Connected:", client.connected) result = await client.read_holding_registers( address=11, count=1, device_id=1 ) print("Before write:", result.registers if not result.isError() else result) write_result = await client.write_register( address=10, value=1234, device_id=1 ) print("Write result:", write_result) result = await client.read_holding_registers( address=10, count=1, device_id=1 ) print("After write:", result.registers if not result.isError() else result) client.close() asyncio.run(main())

# 10. Run the Read/Write Test

Step 17: Execute the Python Client

Run this command from the project directory while the simulator remains running.

py .\test_read_write.py

Step 18: Observe the Successful Output

The successful test produced:

Connected: True Before write: [0] Write result: WriteSingleRegisterResponse(... registers=[1234] ...) After write: [1234]

# 11. What the Test Proves

The test verifies the complete basic communication cycle between the Python client and the simulated PLC.

| Test | Observed Result | Meaning |
| --- | --- | --- |
| Connection | Connected: True | Python successfully connected to the simulated PLC. |
| Initial read | Before write: [0] | A holding register value was successfully read. |
| Write | registers=[1234] | The PLC accepted the write request for register 10. |
| Read after write | After write: [1234] | The written value was stored and returned by the PLC. |